| Initial |  |
| --- | --- |
|  | If you are involved in an accident, DO NOT MOVE the equipment until a supervisor |
|  | is notified and arrives to investigate. |
|  | Failure to report an accident may result in termination. |
|  | Daily checklists must be performed before truck is operated. |
|  | If any problems are found during checklist inspection or throughout the day they |
|  | must be reported to maintenance or a supervisor immediately. |
|  | Any issues must be written on checklist and the supervisor must initial |
|  | checklist showing it was reported. |
|  | A safety harness and lanyard must always be worn while operating a picker/turret. |
|  | A pallet must be on a picker carts forks at all times, this includes while parked. |
|  | Trucks must be lowered completely to the ground before exiting off machine. |
|  | Wire guidance must be fully acquired. This means before entering an aisle, 2 beeps |
|  | must be heard while in the P&D part of the rack. |
|  | Travel at half speed in reverse while backing down a narrow aisle. |
|  | Only 1 truck is allowed in a narrow aisle. |
|  | At no time should you enter an aisle with a cart or any part of your body if it is |
|  | already occupied by another cart. |
|  | You may only pick from the aisle your truck is acquired to wire guidance in. |
|  | You may not walk across skids to pick from an adjacent aisle. |
|  | You may not disconnect from your lanyard at any time in narrow aisles. |
|  | If your truck is stuck in the air and is able to drive to the front or back of an aisle |
|  | call for help and send someone to get a supervisor. Do not exit aisle completely. |
|  | Do not climb down a rack or jump off your Order Picker or Turret Truck if |
|  | it is stuck in a raised position and will not drive. Do your best to signal for help. |
|  | Always turn off your truck when not in use or more than 25 feet away. |
|  | Enter the center aisle of the 100/200 racks on the right side under ENTER sign. |
|  | Exit the center aisle of the 100/200 racks on the opposite side under the EXIT sign. |
|  | Always follow direction arrows while entering/exiting addition. |
|  | Slow down and check globe mirrors above for cross traffic at intersections. |
|  | Report water on floor to supervisors or maintenance and slow down when |
|  | traveling through water. |
|  | Slow down before exiting narrow aisles and look both ways. |
|  | Turn off truck and unplug before changing battery. |
|  | Once the battery is in the changer cart, raise up the guards to keep the |
|  | battery from falling out when moving. |
|  | Secure battery plate snug but DO NOT OVER TIGHTEN. |
|  | Pallets must not be raised above guards while traveling when clearing empties. (Turret ONLY) |