Name: ________________________ Date __/__/__   Correct ___/16_

1. The load handling functions the TSP 6000 can perform are: (Circle all that apply)

- Lift (load and operator)

- Lift (load only)

- Traverse

- Sideshift

- Pivot

- Tilt

- Auto Pivot

2. You must turn the wire guidance on before the truck will acquire the wire and guide the truck automatically.

- True

- False

3. If lit, the wrench light tells the operator: (Circle all that apply)

- The drive tire lug wrench is missing.

- The truck is experiencing a problem.

- You should power the truck down and advise your supervisor or maintenance department.

- You should discontinue use of the truck.

- You should tag the truck out to make sure no one else uses it.

4. The TSP 6000 is made for use on ramps and grades.

- True

- False

5. The guidance system will detect other trucks or personnel in the aisle and/or pallets or boxes protruding from the racking and stop the truck automatically when approaching.

- True

- False

6. The “home position” of the forks is?

- The best position for traveling.

- The fork position required before the truck will raise or lower.

- The full lowered and retracted position for parking the truck.

- The home position for the pallet in the rack.

7. The braking distance at top travel speed is roughly ______ the distance required to brake at one half of top speed.

- the same as

- twice

- 3 times

- 4 times

8. The plugging distance at top travel speed is roughly ______ the distance required to brake at one half of top speed.

- the same as

- twice

- 3 times

- 4 times

9. When standing you apply the brakes by (Pick 2):

- Depressing one of the two smaller foot (brake) pedals.

- Depressing one of the three larger foot (brake) pedals.

- Moving your left foot off one of the larger foot (presence) pedals.

- Moving your right foot off one of the larger foot (presence) pedals.

10. When seated, the brakes are applied by:

- Depressing one of the two smaller foot (brake) pedals.

- Depressing one of the three larger foot (brake) pedals.

- Moving your left foot off one of the larger foot (presence) pedals.

- Moving your right foot off one of the larger foot (presence) pedals.

11. The forks on a TSP600 are adjustable. Why?

- To allow for a bent fork.

- So you can adjust them as far as the load will allow for better load stability.

- So they can be adjusted for wear.

- Only a mechanic should adjust.

- They are not adjustable.

12. Travel speed is reduced when (Pick 2):

- The forks are above freelift.

- Wire guidance is turned off.

- Forks are out of home position.

- You are operating from the standing position.

- When the gates are raised.

13. The TSP 6000 weighs as much as:

- The average car

- Two average cars

- Three average cars

- Four average cars

- None of the above

14. Running over objects on the floor is ok because the truck is so heavy.

- True

- False

15. It’s ok to travel outside the guided aisle with the platform well elevated as you can see better from up high.

- True

- False

16. You should always wear your safety belt or harness and tether when you have a side gate                                                            up on the standard TSP 6000 or at all times when operating a TSP 6000 with a narrow front rail.

- True

- False