Material Safety Data Sheet

Supply Co. - SCRUBS LEATHER CLEANER & PROTECTANT WIPES

Document Nbr: 005710

Class C Parts IDs 905030

[image: image1.png]


ITW DYMON

MATERIAL SAFETY DATA SHEET

HMIS(R*):

HEALTH              1

FLAMMABILITY        1

REACTIVITY          0

PERSONAL PROTECTION NONE

IDENTITY: SCRUBS LEATHER CLEANER & PROTECTANT WIPES

FORMULA: B925

PART NUMBERS: 92501, 92560

DATE PREPARED: 27 OCTOBER 2008

------SECTION 1: COMPANY IDENTIFICATION ------

MANUFACTURER'S NAME: ITW DYMON

ADDRESS (NUMBER, STREET, CITY, STATE, AND ZIP CODE):

805 EAST OLD 56 HIGHWAY

OLATHE, KANSAS 66061

EMERGENCY TELEPHONE NUMBER: 1-800-535-5053

INTERNATIONAL EMERGENCY TELEPHONE NUMBER: 1-352-323-3500

TELEPHONE NUMBER FOR INFORMATION: 1-913-397-9889

SIGNATURE OF PREPARER (OPTIONAL): REGULATORY DEPT.

------SECTION 2: INGREDIENTS/IDENTITY INFORMATION ------

COMPONENTS             CAS NO.     OSHA   ACGIH-    OTHER LIMITS   % (OPT.)

(CHEMICAL IDENTITY,                PEL    TLV       RECOMMENDED

COMMON NAME(S))

WATER                   7732-18-5  NONE   NONE                     40 - 70%

POLYDIMETHYLSILOXANE   63148-62-9  NONE   NONE                     15 - 40%

METHOXYISOPROPANOL       107-98-2  NONE   100 PPM   STEL: 150 PPM   1 - 5%

PROPYLENE GLYCOL          57-55-6  NONE   NONE                      1 - 5%

C12-15 PARETH-7        68181-62-9  NONE   NONE                      0.5 - 1.5%

ANY SUBSTANCE LISTED AS HAZARDOUS BY THE STATES OF CALIFORNIA, FLORIDA,

ILLINOIS, MICHIGAN, NEW JERSEY, OHIO, PENNSYLVANIA OR TEXAS IS DESCRIBED

ABOVE IF KNOWN PRESENT IN REGULATED CONCENTRATIONS.

------SECTION 3: HAZARDS IDENTIFICATION ------

ROUTE(S) OF ENTRY:

EYES?: YES

INHALATION?: NO

SKIN?: YES

INGESTION?: NO

ACUTE HEALTH HAZARDS:

WARNING. DIRECT EYE CONTACT WITH TOWEL CAUSES EYE ABRASION AND IRRITATION.

MAY BE HARMFUL IF SWALLOWED. MAY CAUSE SKIN IRRITATION.

CHRONIC HEALTH HAZARDS:

REPEATED OR PROLONGED OCCUPATIONAL OVEREXPOSURES MAY CAUSE DERMATITIS,

CENTRAL NERVOUS SYSTEM EFFECTS, KIDNEY EFFECTS OR PAINTER'S SYNDROME.

CARCINOGENICITY:

CHEMICAL: NONE

OSHA: NO

NTP: NO

IARC: NO

SIGNS AND SYMPTOMS OF EXPOSURE:

REDNESS, TEARING OR BURNING IN EYES. REDNESS, DRYING OR CRACKING SKIN.

IRRITATION OF THE THROAT OR STOMACH, NAUSEA, VOMITING, AND DIARRHEA IF

SWALLOWED. OVEREXPOSURE TO VAPORS (UNLIKELY) MAY CAUSE CENTRAL NERVOUS

SYSTEM EFFECTS SUCH AS DROWSINESS, DIZZINESS, FATIGUE OR LOSS OF

COORDINATION.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE:

PRE-EXISTING SKIN CONDITIONS SUCH AS DERMATITIS MAY BE ADVERSELY AFFECTED

BY THIS AND OTHER OIL AND GREASE EFFECTIVE CLEANERS. PERSONS WITH

PRE-EXISTING LIVER AND KIDNEY CONDITIONS SHOULD FOLLOW GOOD CHEMICAL

HYGIENE PRACTICES TO AVOID COMPLICATIONS.

------SECTION 4: EMERGENCY AND FIRST AID PROCEDURES ------

EYES:

FLUSH WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES LIFTING EYELIDS TO

INSURE COMPLETE REMOVAL. SEE A PHYSICIAN IMMEDIATELY.

INGESTION:

NOT A LIKELY EXPOSURE ROUTE. IF A LARGE QUANTITY OF LIQUID IS SWALLOWED,

DO NOT INDUCE VOMITING UNLESS DIRECTED BY A PHYSICIAN. NEVER GIVE ANYTHING

BY MOUTH TO AN UNCONSCIOUS PERSON. CALL A PHYSICIAN OR POISON CONTROL

CENTER IMMEDIATELY.

INHALATION:

UNLIKELY ROUTE AS LIQUID IS IMPREGNATED ON A TOWEL, MINIMIZING EXPOSURE VIA

THIS ROUTE. IF OVEREXPOSURE OCCURS, GET TO FRESH AIR. IF BREATHING HAS

STOPPED, QUALIFIED PERSONNEL SHOULD ADMINISTER ARTIFICIAL RESPIRATION.

SKIN:

FLUSH WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES WHILE REMOVING

CONTAMINATED CLOTHING. IF IRRITATION ARISES AND PERSISTS, CALL A PHYSICIAN.

------SECTION 5: FIRE FIGHTING MEASURES ------

FLASH POINT CONCENTRATE (METHOD USED): NONE TO BOILING (PMCC)

FLAMMABLE LIMITS: NO DATA

LEL: NO DATA

UEL: NO DATA

EXTINGUISHING MEDIA:

NOT USUALLY NECESSARY AS THIS PRODUCT DOES NOT SUPPORT COMBUSTION. USE

MEDIA APPROPRIATE FOR FIRE'S FUEL SOURCE.

SPECIAL FIRE FIGHTING PROCEDURES:

KEEP CONTAINERS COOL AND VAPORS DOWN WITH WATER SPRAY. PREVENT RUNOFF FROM

ENTERING SEWERS AND PUBLIC WATERWAYS. WEAR SELF-CONTAINED BREATHING

APPARATUS (SCBA) IN CHEMICAL FIRES.

HAZARDOUS PRODUCTS OF COMBUSTION:

CARBON DIOXIDE, CARBON MONOXIDE, SULFUR DIOXIDE, SILICONE DIOXIDE, HYDROGEN

SULFIDE, FORMALDEHYDE (>300 DEG. F), SMOKE, SOOT, AND VARIOUS ORGANIC

OXIDATION BY-PRODUCTS.

UNUSUAL FIRE AND EXPLOSION HAZARDS:

NONE KNOWN. IF PLASTIC MATERIAL IS IGNITED, IT IS DIFFICULT TO EXTINGUISH.

------SECTION 6: ACCIDENTAL RELEASE MEASURES ------

SMALL SPILL:

CAUTION, SLIP HAZARD. WIPE UP SMALL RELEASES WITH A DRY ABSORBENT CLOTH OR

OTHER ABSORBENT MATERIAL.

LARGE SPILL:

ABSORB LIQUID WITH VERMICULITE, ABSORBENT CLOTH, OR OTHER ABSORBENT

MATERIAL. PREVENT MATERIAL FROM ENTERING SEWERS OR DRAINS. VENTILATE AREA

AND BLOCK TRAFFIC. TRANSFER CONTAMINATED MATERIAL INTO SUITABLE CONTAINER

FOR PROPER DISPOSAL. CLEAN ANY SLIPPERY RESIDUES WITH SOAP AND WATER.

------SECTION 7: HANDLING AND STORAGE ------

HANDLING:

DO NOT ALLOW TOWEL CONTACT WITH EYES. WASH THOROUGHLY AFTER HANDLING. AVOID

SWALLOWING. AVOID CONTACT WITH SKIN AND CLOTHING. AVOID INHALING VAPORS.

USE WITH ADEQUATE VENTILATION. DO NOT SMOKE WHILE USING.

STORAGE:

FOLLOW LABEL DIRECTIONS CAREFULLY. KEEP OUT OF REACH OF CHILDREN. STORE IN

COOL WELL VENTILATED AREA. KEEP AWAY FROM HEAT SOURCES. KEEP CONTAINER

TIGHTLY SEALED WHEN NOT IN USE. DO NOT CONTAMINATE WATER, FOOD OR FEED BY

USE OR STORAGE. KEEP FROM FREEZING.

------SECTION 8: EXPOSURE CONTROLS / PERSONAL PROTECTION ------

EYE PROTECTION: NOT USUALLY NECESSARY.

SKIN PROTECTION:

NOT USUALLY NECESSARY. IF REPEATED OR PROLONGED CONTACT IS POSSIBLE, WEAR

IMPERVIOUS GLOVES.

RESPIRATORY PROTECTIONS:

NOT USUALLY NECESSARY. USE WITH ADEQUATE VENTILATION. USE NIOSH/MSHA

APPROVED RESPIRATOR IF PELS OR TLVS ARE EXCEEDED.

ENGINEERING CONTROLS:

VENTILATION NOT USUALLY NECESSARY BUT SHOULD BE PROVIDED IN THE EVENT OF

OVEREXPOSURE.

------SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES ------

APPEARANCE AND ODOR:

THIN, WHITE OPAQUE LIQUID WITH MINIMAL SCENT SATURATED ONTO TOWELS.

pH: 7.7 +/- 0.5

SPECIFIC GRAVITY (70 DEG. F): 0.995

BOILING POINT: 212 DEG. F

MELTING POINT: NO DATA

EVAPORATION RATE (BUTYL ACETATE = 1): NO DATA

VAPOR PRESSURE (MM-Hg @ 70 DEG. F): NO DATA

VAPOR DENSITY (AIR = 1): >1

SOLUBILITY IN WATER: DISPERSIBLE

VOLATILE ORGANIC COMPOUNDS (VOCS): 2.95% BY WEIGHT

------SECTION 10: STABILITY AND REACTIVITY ------

HAZARDOUS POLYMERIZATION:

THIS PRODUCT WILL NOT UNDERGO HAZARDOUS POLYMERIZATION.

HAZARDOUS DECOMPOSITION OR BYPRODUCTS:

CARBON DIOXIDE, CARBON MONOXIDE, SULFUR DIOXIDE, SILICONE DIOXIDE, HYDROGEN

SULFIDE, FORMALDEHYDE (>300 DEG. F), SMOKE, SOOT, AND VARIOUS ORGANIC

OXIDATION BY-PRODUCTS.

CHEMICAL STABILITY: STABLE

INCOMPATIBILITY (MATERIALS TO AVOID):

STRONG ACIDS AND BASES, STRONG OXIDIZERS, LIME, AMMONIA, ORGANIC AMINES,

CHLORATES, CHLORINE, SODIUM HYDROXIDE.

------SECTION 11: TOXICOLOGICAL INFORMATION ------

NO DATA

------SECTION 12: ECOLOGICAL INFORMATION ------

NO DATA

------SECTION 13: DISPOSAL CONSIDERATION ------

DISPOSAL METHOD:

DISPOSE OF IN ACCORDANCE WITH ALL APPLICABLE LOCAL, STATE, AND FEDERAL

REGULATIONS. 

RCRA WASTE INFORMATION:

IF THIS PRODUCT BECOMES A WASTE, IT WOULD NOT BE A HAZARDOUS WASTE AS

DEFINED BY RCRA (40 CFR 261). HOWEVER THE WASTE SHOULD BE PROPERLY

CHARACTERIZED TO EVALUATE WHETHER ITS COMPOSITION HAS BEEN MODIFIED PRIOR

TO DISPOSAL.

------SECTION 14: TRANSPORT INFORMATION ------

DOT INFORMATION - 49 CFR 172.101:

PROPER SHIPPING DESCRIPTION: NOT REGULATED

HAZARD CLASS: NONE

IDENTIFICATION NUMBER: NONE

PACKING GROUP: NONE

WATER TRANSPORTATION:

PROPER SHIPPING DESCRIPTION: NOT REGULATED

HAZARD CLASS: NONE

IDENTIFICATION NUMBER: NONE

PACKING GROUP: NONE

AIR TRANSPORTATION:

PROPER SHIPPING DESCRIPTION: NOT REGULATED

HAZARD CLASS: NONE

IDENTIFICATION NUMBER: NONE

PACKING GROUP: NONE

MARINE POLLUTANTS: NONE

------SECTION 15: REGULATORY INFORMATION ------

US FEDERAL REGULATIONS:

TSCA (TOXIC SUBSTANCES CONTROL ACT) STATUS:

ALL INGREDIENTS ARE ON THE TSCA INVENTORY OR ARE NOT REQUIRED TO BE LISTED

ON THE TSCA INVENTORY.

CERCLA RQ - 40 CFR 302.4 (A): NONE 

SARA 313 COMPONENTS - 40 CFR 372.65: NONE

STATE AND LOCAL REGULATIONS:

CALIFORNIA PROPOSITION 65: THIS PRODUCT CONTAINS A TRACE CHEMICAL 

(ACETALDEHYDE) KNOWN TO THE STATE OF CALIFORNIA TO CAUSE CANCER AND/OR BIRTH 

DEFECTS OR OTHER REPRODUCTIVE HARM.

WARNING:

THIS PRODUCT CONTAINS TRACE CHEMICALS (ETHYLENE OXIDE, ACETALDEHYDE AND

PROPYLENE OXIDE) KNOWN TO THE STATE OF CALIFORNIA TO CAUSE CANCER, AND/OR

BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM.

------SECTION 16: ADDITIONAL INFORMATION ------

WARNING! THE USE OF THIS PRODUCT IS BEYOND THE CONTROL OF THE MANUFACTURER;

THEREFORE, NO GUARANTEE, EXPRESSED OR IMPLIED, IS MADE AS TO THE EFFECTS

OF SUCH OR THE RESULTS TO BE OBTAINED IF NOT USED IN ACCORDANCE WITH

DIRECTIONS OR ESTABLISHED SAFE PRACTICE. THE USER MUST ASSUME ALL

RESPONSIBILITY, INCLUDING INJURY OR DAMAGE, RESULTING FROM ITS MISUSE AS

SUCH, OR IN COMBINATION WITH OTHER MATERIALS. THE MANUFACTURER WARRANTS

ONLY THAT THIS PRODUCT MEETS THE MANUFACTURER'S SPECIFICATIONS FOR SUCH

PRODUCT. THIS WARRANTY IS IN LIEU OF ALL OTHER WARRANTIES, EXPRESS OR

IMPLIED, AS TO DESCRIPTION, QUALITY, MERCHANTABILITY. FITNESS FOR ANY

PARTICULAR PURPOSE, PRODUCTIVENESS, OR ANY OTHER MATTER, OF THIS PRODUCT.

THE MANUFACTURER SHALL BE IN NO WAY RESPONSIBLE FOR THE IMPROPER USE OF

THIS PRODUCT. THE SOLE AND EXCLUSIVE REMEDY AGAINST THE MANUFACTURER FOR

BREACH OF WARRANTY SHALL BE REIMBURSEMENT OF THE PURCHASE PRICE OF THE

PRODUCT IN THE EVENT THAT A DEFECTIVE CONDITION OF THE PRODUCT SHALL BE

FOUND TO EXIST. NO OTHER REMEDY (INCLUDING BUT NOT LIMITED TO INCIDENTAL

OR CONSEQUENTIAL DAMAGES FOR INJURY TO PERSON OR PROPERTY OR ANY OTHER

INCIDENTAL OR CONSEQUENTIAL LOSS) SHALL BE AVAILABLE.

